Recovery of the building heritage through the NEB lens

1. In refurbishment interventions of reinforced concrete frame buildings with CLT panel systems:

- Connections between panels and the existing structure are distributed at regular intervals across the entire panel surface.

- The panels form an external envelope around the original structure.

- The panels are fixed on the inside of the existing structures.

- During panel installation, activities carried out inside the existing building must be halted.

2. In refurbishment interventions of masonry buildings with CLT panel systems:

- They allow the building to be upgraded in terms of seismic and energy performance, but not in terms of functionality.

- They are carried out exclusively using chemical anchors for the connection between CLT panels and masonry.

- They are particularly suitable for buildings with historical-architectural constraints limited to the external façades.

- They do not allow the use of widespread prefabrication techniques.

3. In a “façadism” intervention carried out using CLT panel construction systems:

- The existing masonry is relieved from the loads of the original horizontal structure

- No interventions on the foundation structures are required

- It is not possible to create additional underground floors

- CLT panels are normally anchored to the existing masonry starting from the level of the new first-floor slab.

4. The ground connection detail in a masonry building refurbishment with CLT panel systems:

- Does not affect the durability of the timber elements

- Must ensure the correct transition between systems with different execution tolerances

- Must consider only the rising damp through the reinforced concrete structures possibly in contact with the timber elements.

- Must primarily meet structural requirements.

5. To ensure proper durability of the CLT panel at the ground connection, it is most important to:

- Protect the inner base of the wall with a waterproof membrane.

- Protect the outer base of the wall with a waterproof membrane.

- Include an EPDM membrane between the foundation and the panel.

- Set an installation height sufficiently above ground level.

6. In which of the following types of foundation-to-CLT panel connections is a leveling screed with thixotropic mortar not necessary?

- With an interposed timber root beam.

- With an in-situ cast reinforced concrete beam with lost formwork.

- With direct planar installation.

- With a traditional reinforced concrete beam.

7. In which of the following types of foundation-to-CLT panel connections is the panel installation height lower than the internal floor level?

- With an interposed aerated metal beam.

- On a leveling screed.

- With an interposed reinforced concrete beam.

- Direct planar installation.

8. In which of the following types of foundation-to-CLT panel connections with an interposed element is the panel installed before the beam?

- With adjustable steel piers.

- With a timber root beam.

- With an aluminum beam.

- With a reinforced concrete beam with lost formwork.